CBAM MONITORING PLAN

Carbon Border Adjustment Mechanism

Commission Implementing Regulation (EU) 2025/2547

Sector: Iron & Steel / Crude Steel

| Installation Name | [Enter Installation Name] |
| --- | --- |
| Country | [Enter Country] |
| Version Number | [e.g. 01] |
| Date of First Creation | [DD/MM/YYYY] |
| Prepared By | [Name / Organisation] |

HOW TO USE THIS TEMPLATE:

1. Text in [square brackets] and italic grey text = fields you must fill in. Delete the brackets after filling.

2. Yellow boxes (like this one) contain instructions. DELETE all yellow guidance boxes before submitting.

3. Green boxes contain filled examples based on a sample steel plant. Use these as reference, then replace with your data.

4. Every section is mandatory under EU 2025/2547. If a section is not applicable to your installation, write 'Not Applicable' and explain why.

5. Attach the System Boundary Diagram (Annexure I) and Precursor Supplier Data as separate documents referenced herein.

# Section 1: Installation Details

[Req. 1] Date and version number of the monitoring plan

[Req. 2] Description of the installation and production processes

## 1.1 Document Control

| Field | Information |
| --- | --- |
| Date of First Creation | 01/01/2026 |
| Version Number | 01 |
| Latest Revision Date | [DD/MM/YYYY] |
| Revision History | [e.g. Version 01 — Initial submission] |

Update the version number every time you make a material change to the monitoring plan.

Format: Version 01, 02, 03... Each version must be dated.

Keep a brief revision history entry for each version — e.g. 'Version 02: Updated EF source from IPCC 2006 to IPCC 2019'.

## 1.2 Installation Identification

| Field | Details |
| --- | --- |
| Name of Installation | [Enter full legal name of installation] |
| Full Address | [Enter complete address including pin/postal code, city, state, country] |
| Latitude & Longitude of location | [Enter the Latitude and Longitude of the installation] |
| Economic Activity | [e.g. Iron & Steel Manufacturing / Crude Steel Production] |
| Authorized Representative Name | [Enter Name] |
| Email | [Enter Email] |
| Contact Number | [Enter Contact Number] |

## 1.3 Description of Production Process

Describe the end-to-end production process at your installation in narrative form.

Cover: (a) key process stages in order, (b) key equipment at each stage with capacity, (c) how each stage connects to the next.

For steel plants: typically SMS (EAF/AOD/LRF/CCM) → Hot Rolling → Heat Treatment → downstream finishing.

Be specific about capacities (MT/hr, MT/batch, etc.) — verifiers use this to cross-check your activity data.

If you use job-work at external installations for any step, explicitly identify those installations here.

Describe your production process below:

EXAMPLE —Production Process Description

The installation operates an integrated steel manufacturing process:

Steel Melting Shop (SMS): Single production line, avg. capacity 45 MT per heat.

- Electric Arc Furnace (EAF): Melts scrap and ferro-alloys

- Argon Oxygen Decarburisation (AOD): Refines steel composition

- Ladle Refining Furnace (LRF): Final composition adjustment

- Continuous Casting Machine (CCM): Casts liquid steel into billets

Hot Rolling Mill: Single line for black bars and wire rods, capacity 15 MT/hr.

Heat Treatment: 10 furnaces (various capacities: 2–28 MT) for black bars, wire rods.

Wire Rod Route: Pickling (2 lines) → Wire Manufacturing (11 multi-die machines + 4 annealing furnaces).

Black Bar / Bright Bar Route: Direct dispatch or peeling/drawing to bright bars.

Note: Heat treatment of black bars for job-work is outsourced to external installations using PNG fuel.

[INSTRUCTION: Replace the example above with your own installation's production process description. Delete this note after filling.]

## 1.4 System Boundary Diagram

[Reg. Req. Req. 13] System boundary diagram with evidence of no double-counting or data gaps

A process flow diagram is NOT sufficient. The system boundary diagram must show:

(a) Physical boundaries of the entire installation

(b) Boundaries of each individual production process (e.g. SMS, Rolling, Heat Treatment)

(c) All inputs entering each production process (fuels, raw materials, electricity, heat)

(d) All outputs leaving each production process (products, precursors transferred internally, waste gases, exported heat)

(e) Explicit annotation confirming no double-counting between processes

(f) Explicit annotation confirming no data gaps (i.e. all emission-relevant flows are captured)

This diagram MUST be attached as Annexure I. Simply referring to a generic process flow will be rejected by verifiers.

Tip: Draw a box for each production process. Draw arrows for every material/energy flow. Label each arrow with what it is.

System Boundary Diagram is attached as Annexure I to this Monitoring Plan. Refer instructions and provide System boundary diagram as per example.

# Section 2: CBAM Goods, Production Processes and Benchmarks

[Req. 3] List of all relevant CBAM goods by CN code and functional unit

[Req. 4] List of all CBAM production processes and routes

[Req. 5] List of non-CBAM goods produced

[Req. 6] List of CBAM benchmarks for free allocation determination

## 2.1 List of CBAM Goods Produced

List every good produced at your installation that falls under CBAM (Annex I of EU 2023/956 and EU 2025/2547).

CN Code: Use the 8-digit Combined Nomenclature code. Verify against Annex I of EU 2025/2547.

Functional Unit: For steel sector this is always 'Tonne (t)'.

Precursor — Internal Transfer: A CBAM good/intermediate from your own installation used as input in another process here.

Precursor — Purchased: A CBAM good bought externally and used as input.

If you are unsure whether a product is CBAM-covered, check the CN code at: https://eur-lex.europa.eu/legal-content/EN/TXT/?uri=OJ:L_202502547

| Sr. | Good Name | Funct. Unit | CN Code | Production Process & Route | Precursor (Internal) | Precursor (Purchased) |
| --- | --- | --- | --- | --- | --- | --- |
| HOW TO FILL: Add one row per CBAM good. For CN Code, use 8-digit code from EU 2025/2547 Annex I. If no precursor, write 'None'. |  |  |  |  |  |  |
| 1 | Billet | Tonne | 72189911 | Produced in SMS via EAF → AOD → LRF → CCM | None | FeCr, FeNi, FeMn |
| 2 | Black Bars | Tonne | 72221119 | Billets → Hot Rolling → Heat Treatment | Billets | None |
| 3 | Bright Bars | Tonne | 72222019 | Black Bars → Heat Treatment → Peeling/Drawing | Black Bars | None |
| 4 | Wire Rod | Tonne | 72210090 | Billets → Hot Rolling → Heat Treatment → Pickling | Billets | None |
| 5 | Wire | Tonne | 72230091 | Wire Rod → Heat Treatment → Pickling → Wire Drawing | Wire Rod | None |
| 6 | [Add row if needed] |  |  |  |  |  |

## 2.2 Non-CBAM Goods Produced

List any goods produced at this installation that are NOT covered by CBAM.

Examples for a steel plant: product with CN code not listed in CBAM list. Check here: Implementing regulation - EU - 2025/2547 - EN - EUR-Lex

If no non-CBAM goods are produced, write 'None' in the table below with a brief explanation.

Why this matters: Non-CBAM goods can share production processes with CBAM goods, affecting emission allocation.

| Sr. | Non-CBAM Good Name | Production Process (shared with which CBAM process) | Quantity Produced (MT/year) |
| --- | --- | --- | --- |
| HOW TO FILL: If no non-CBAM goods: write 'None — all goods produced at this installation are CBAM-covered.' Delete unused rows. |  |  |  |
| 1 | [e.g. Mill Scale] | [e.g. Hot Rolling — shared with Black Bars and Wire Rod] | [e.g. ~500 MT/year] |
| 2 |  |  |  |

## 2.3 CBAM Benchmarks for Free Allocation

For each CBAM good, two benchmark values apply (from EU 2025/2620):

BMg* (BMg-star): Process-related benchmark — used when the precursor's Specific Embedded Emissions (SEE) are calculated.

BMg (BMg): Default benchmark — used when the precursor SEE is taken as the CBAM default value.

Source: Commission Implementing Regulation (EU) 2025/2620. Verify values against the latest published version.

These benchmarks are used by EU declarants to determine any free allocation adjustment — not for your emissions calculation directly. These numbers vary every year; hence year is necessary.

| Sr. | Good Name | CN Code | BMg* (tCO2e/t) — Precursor SEE Calculated | BMg (tCO2e/t) — Precursor SEE Default | Source |
| --- | --- | --- | --- | --- | --- |
| HOW TO FILL: Enter benchmark values from EU 2025/2620. Do not leave blank — if value is 0, write 0 and note why. |  |  |  |  |  |
| 1 | Billet | 72189911 | 0.128 | 1.189 | EU 2025/2620 |
| 2 | Black Bars | 72221119 | 0.109 | 1.225 | EU 2025/2620 |
| 3 | Bright Bars | 72222019 | 0.109 | 1.225 | EU 2025/2620 |
| 4 | Wire Rod | 72210090 | 0.109 | 1.225 | EU 2025/2620 |
| 5 | Wire | 72230091 | 0.109 | 1.225 | EU 2025/2620 |

# Section 3: Monitoring Methodology — Direct (Scope 1) Emissions

[Req. 7] Methods for monitoring data per production process

[Req. 8] Calculation factors and sampling plan per source stream

[Req. 9] List of source streams and emission sources

[Req. 10] Tier levels for calculation-based / mass balance methods (B.3.4)

[Req. 11] Emission sources using measurement-based methodology (CEMS)

## 3.1 Overview of Methodology Applied

For each production process, state which methodology is used to calculate direct (Scope 1) emissions.

Two methodologies are available under EU 2025/2547:

(A) Calculation-Based: Activity Data × Calculation Factor (NCV × EF) — used for fuel combustion.

(B) Mass Balance: (Carbon In − Carbon Out) × 44/12 — used for carbon-bearing raw materials (scrap, ferro-alloys, etc.).

Most steel plants use BOTH methods — combustion fuels use (A), carbon-bearing materials use (B).

You must specify which tier (Tier 1, 2, or 3) is used for each parameter under B.3.4 of the Regulation.

Tier 1 = lowest accuracy (default values), Tier 3 = highest (lab analysis). Use the highest tier you can achieve. While emission sources having less contribution to emission can use Tier 1 but sources having higher contribution should use Tire 2 & 3. Refer to sampling frequency requirement and laboratory qualification

| Production Process | Methodology Applied | Combustion Fuels Used | Carbon-Bearing Materials |
| --- | --- | --- | --- |
| SMS — Billet production | Calculation-based (combustion) + Mass Balance | LSHS, LPG, LDO | MS Scrap, SS Scrap, FeCr, FeNi, FeMn, Lime, Dolomite, Pet Coke |

Mention attribute related description here.

## 3.2 Calculation-Based Methodology — Combustion Source Streams

Tier System for Combustion Parameters — What You Must State:

Activity Data Tier: Tier 1 = estimated/no dedicated meter. Tier 2 = metered but lower-accuracy instrument. Tier 3 = metered with calibrated instrument traceable to national standard (NABL/NPL). For major fuels (LSHS, LDO), aim for Tier 2 minimum. Theoretical calculation with no meter is Tier 1 and must be justified — verifiers will flag it as a weakness.

NCV Tier: Tier 1 = IPCC published default (e.g. LSHS = 40.2 GJ/MT). Tier 2 = country/industry-specific published value (e.g. BEE India). Tier 3 = in-house lab analysis per delivery lot. State which table/publication your default value comes from — "Default IPCC" alone is not enough; cite the specific table and year (e.g. IPCC 2006 GL, Volume 2, Table 1.2).

EF Tier: Tier 1 = IPCC default EF. Tier 2 = country-specific published EF. Tier 3 = lab-determined per fuel batch. For standard fossil fuels, Tier 1 EF is accepted by verifiers. State the specific IPCC value used (e.g. LPG EF = 0.0631 tCO2/GJ) — do not just write "IPCC default."

Why tiers matter: The regulation requires you to use the highest tier that is technically feasible and not unreasonably costly. If you use Tier 1 for a major fuel stream, you should briefly note why a higher tier is not applied.

Tier System for Combustion Parameters — What You Must State:

Activity Data Tier: Tier 1 = estimated/no dedicated meter. Tier 2 = metered but lower-accuracy instrument. Tier 3 = metered with calibrated instrument traceable to national standard (NABL/NPL). For major fuels (LSHS, LDO), aim for Tier 2 minimum. Theoretical calculation with no meter is Tier 1 and must be justified — verifiers will flag it as a weakness.

NCV Tier: Tier 1 = IPCC published default (e.g. LSHS = 40.2 GJ/MT). Tier 2 = country/industry-specific published value (e.g. BEE India). Tier 3 = in-house lab analysis per delivery lot. State which table/publication your default value comes from — "Default IPCC" alone is not enough; cite the specific table and year (e.g. IPCC 2006 GL, Volume 2, Table 1.2).

| Source Stream | Production Process | Activity Data — How Measured | Activity DaMentMFrequency | NCV (Default/Measured) + Tier | EF (Default/Measured) + Tier | Emission Formula | Responsible Person |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HOW TO FILL: For each row: state the exact measurement method, instrument name, location, and frequency. 'Theoretical calculation' is only acceptable if no meter is available — and must be justified. |  |  |  |  |  |  |  |
| LSHS (MT) | SMS — Billet (Ladle preheating) | Scale-tube level meter on sub-tank. Formula: π × R² × (Initial Level − Final Level) × Density | Daily | Default IPCC — Tier 1. NCV = 40.2 GJ/MT | Default IPCC — Tier 1. EF = 0.0795 tCO2/GJ | AD × NCV × EF | SMS Maintenance Fitter |
| LSHS (MT) | Hot Rolling (Billet reheating) | Scale-tube level meter on main tank. SMS consumption deducted. | Daily | Default IPCC — Tier 1. NCV = 40.2 GJ/MT | Default IPCC — Tier 1. EF = 0.0795 tCO2/GJ | AD × NCV × EF | RMD Fitter |
| LPG (MT) | SMS — Billet (Ladle/vessel/tundish preheating) | Theoretical: Total LPG − RMD − HT − Wire sections. No physical meter installed. | Daily | Default IPCC — Tier 1. NCV = 47.3 GJ/MT | Default IPCC — Tier 1. EF = 0.0631 tCO2/GJ | AD × NCV × EF | GM SMS Maintenance |
| LPG (MT) | Hot Rolling (Cutting) | Theoretical: 0.07 kg/MT of production | Daily | Default IPCC — Tier 1 | Default IPCC — Tier 1 | AD × NCV × EF | [Enter name] |
| LPG (MT) | Heat Treatment | Theoretical per furnace: SAF=40 kg/MT; BHF2&3=33; BHF4=67; BHF5&6=48 kg/MT | Daily | Default IPCC — Tier 1 | Default IPCC — Tier 1 | AD × NCV × EF | [Enter name] |
| LPG (MT) | Wire Manufacturing (Annealing) | Rotameter reading × 54 (conversion factor). Recorded daily in logbook → SAP. | Daily | Default IPCC — Tier 1 | Default IPCC — Tier 1 | AD × NCV × EF | Wire Fitter |
| LDO (MT) | SMS — Billet (Ladle preheating) | Scale-tube level meter on SMS tank. | Daily | Default IPCC — Tier 1. NCV = 43.0 GJ/MT | Default IPCC — Tier 1. EF = 0.0741 tCO2/GJ | AD × NCV × EF | SMS Production Fitter |
| LDO (MT) | Heat Treatment — Wire Rod | Scale-tube level meter on HT tank. | Daily | Default IPCC — Tier 1 | Default IPCC — Tier 1 | AD × NCV × EF | HT Operator |
| PNG (MT) | Heat Treatment — Black Bars (Job-work, external) | Collected from external job-work contractor monthly. Monthly consumption certificate required. | Monthly from contractor | Default IPCC — Tier 1 | Default IPCC — Tier 1 | AD × NCV × EF | [Enter responsible person] |
| [Add fuel] |  |  |  |  |  |  |  |

## 3.3 Mass Balance Methodology — Carbon-Bearing Materials

Used for: all carbon-bearing input materials (scrap, ferro-alloys, fluxes, pet coke) AND carbon-bearing output materials (liquid steel/billets, slag).

Formula: Emissions (tCO2) = [Sum of (AD_input × CC_input) − Sum of (AD_output × CC_output)] × 44/12

where AD = Activity Data (MT), CC = Carbon Content (t-carbon/t-material)

Carbon Content: Can be Default (published reference) or Measured (in-house lab using Optical Spectrometer or LECO method).

In-house measurement is Tier 2/3 and preferred. State which instrument is used and sampling frequency.

LECO combustion analysis: Determines C, H, N, O, S content. Used for ferro-alloys and scraps.

Optical Spectrometer: Analyses polished samples by light dispersion. Used for scrap and output steel.

Gravimetric analysis: Mass-based method. Used for pet coke carbon content.

Sampling plan: State how many samples per truck/batch and how you ensure representativeness.

| Source Stream | Input / Output | Activity Data — How Measured | AD Frequency | Carbon Content — Method + Tier | Sampling Plan | Formula | Responsible — CC Analysis |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HOW TO FILL: Positive AD = input (adds carbon to process). Negative AD = output (removes carbon). Net carbon × 44/12 = CO2. |  |  |  |  |  |  |  |
| Mild Steel Scrap | INPUT (+) | Weighed on weighbridge at reception. Recorded in SAP. | Per truck on receipt | In-house Optical Spectrometer — Tier 2. Each truck sampled. | 1 sample per truck; number varies by scrap homogeneity | AD × CC × 44/12 | Lab Incharge |
| Stainless Steel Scrap | INPUT (+) | Weighed on weighbridge at reception. Recorded in SAP. | Per truck on receipt | Optical Spectrometer + LECO Method — Tier 2/3 | 1 sample per truck | AD × CC × 44/12 | Lab Incharge |
| Ferro Nickel | INPUT (+) | Weighed on weighbridge at reception. | Per truck | LECO Method — Tier 3. Every truck: 3 bags from different locations. | 3 bags per truck | AD × CC × 44/12 | Lab Incharge |
| Ferro Chrome | INPUT (+) | Weighed on weighbridge at reception. | Per truck | LECO Method — Tier 3. Every truck: 3 bags. | 3 bags per truck | AD × CC × 44/12 | Lab Incharge |
| Ferro Manganese | INPUT (+) | Weighed on weighbridge at reception. | Per truck | LECO Method — Tier 3. Every truck: 3 bags. | 3 bags per truck | AD × CC × 44/12 | Lab Incharge |
| Other Ferro Alloys | INPUT (+) | Weighed on weighbridge. | Per truck | LECO Method — Tier 3. | 3 bags per truck | AD × CC × 44/12 | Lab Incharge |
| Pet Coke | INPUT (+) | Weighed at FAAS system. Recorded heat-wise in SAP. | Heat-wise | Gravimetric Analysis — Tier 2. 3 bags per truck. | 3 bags per truck | AD × CC × 44/12 | Lab Incharge |
| Lime | INPUT (+) | Weighed at FAAS conveyor. Recorded heat-wise in SAP. | Heat-wise | LECO Method — Tier 3. 3 bags per truck. | 3 bags per truck | AD × CC × 44/12 | Lab Incharge |
| Dolomite | INPUT (+) | Weighed at FAAS conveyor. Recorded heat-wise in SAP. | Heat-wise | LECO Method — Tier 3. 3 bags per truck. | 3 bags per truck | AD × CC × 44/12 | Lab Incharge |
| Skull (returned scrap) | INPUT (+) | Weighed at SMS weighbridge. | Heat-wise | LECO Method — Tier 3. | N/A (internal material) | AD × CC × 44/12 | Lab Incharge |
| Liquid Steel / Billets (output) | OUTPUT (−) | Weighed at billet yard weighing scale. Recorded heat-wise in SAP. | Heat-wise | Optical Spectrometer + LECO — Tier 2/3. Heat-wise sampling. | Per heat | AD × CC × 44/12 | Lab Incharge |
| Slag (output) | OUTPUT (−) | Quantity estimated / [Enter method if weighed] | [Frequency] | [Enter CC method] | [Enter sampling] | AD × CC × 44/12 | Lab Incharge |
| [Add material] |  |  |  |  |  |  |  |

## 3.4 Measurement-Based Methodology (CEMS)

This section applies only if your installation uses Continuous Emission Monitoring Systems (CEMS) on any stack.

If no CEMS are installed, write the statement below and this section is complete.

If CEMS are used, describe: stack ID, pollutant measured, instrument make/model, location, calibration frequency, and data handling. Else, write the following:

No measurement-based methodology (CEMS) is used at this installation. All emissions are calculated using the calculation-based and mass balance methodologies described in Sections 3.2 and 3.3 above.

# Section 4: Indirect Emissions — Electricity (Scope 2)

[Req. 17] Whether electricity is produced inside the installation

[Req. 18 & 19] Basis for indirect emission determination and elements of evidence

## 4.1 Electricity Generation at the Installation

Answer all four sub-questions below. These are mandatory regardless of sector.

These questions apply to ALL CBAM sectors — not just electricity producers.

The purpose is to determine whether any electricity produced on-site exits the production process boundary (which would affect emission attribution).

If you have any captive power plant, DG sets, co-gen units, or solar panels — you must disclose them here.

| Question (Reg. Req. 17) | Response |
| --- | --- |
| (a) Is electricity produced by co-generation inside the installation? | NO — No co-generation unit is installed at this installation. |
| (b) Is electricity produced by separate generation (e.g. captive power, DG set)? | NO — No captive power plant or DG set is used for production. [Edit if applicable — if yes, describe installed capacity, fuel used, % of own consumption supplied] |
| (c) Is the electricity produced from fossil or renewable sources? | N/A — No electricity is produced at the installation. All electricity is sourced externally from the state grid. |
| (d) Is any electricity exported from the system boundary of a production process? | NO — No electricity is exported from any production process boundary. |

## 4.2 Indirect Emission Monitoring — Electricity

For each production process, document: (a) how electricity consumption is measured, (b) the emission factor used to convert kWh to tCO2e.

Emission Factor: For electricity imported from the Indian grid, the default emission factor published by the Central Electricity Authority (CEA) of India is used.

State the exact value and year of the factor used. Example: CEA 2022-23 grid EF = 0.716 tCO2/MWh (verify current figure).

If using ACTUAL emission factor (not default): you must provide evidence per Annex D.2.4 of EU 2025/2547 — including the electricity source, generation mix, and supporting documentation.

Most non-EU plants will use the default emission factor. State this explicitly.

| Product | Production Process | How Electricity Consumption is Measured | Frequency | Emission Factor Applied (tCO2/MWh) | EF Source & Basis |
| --- | --- | --- | --- | --- | --- |
| HOW TO FILL: Fill in the EF value you are using. If using CEA default, state 'CEA [year] default = [value]'. Do not leave blank. |  |  |  |  |  |
| Billet | SMS | Sub-meter reading. Formula: (Current − Previous Reading) × Multiplier Factor. Recorded daily in SAP. | Daily | [e.g. 0.716 tCO2/MWh] | CEA India Grid EF [state year]. Default value — actual emissions not used. |
| Black Bars | Hot Rolling | Sub-meter reading — RMD department. | Daily | [same as above] | CEA India Grid EF [state year] |
| Black Bars | Heat Treatment | Sub-meter reading — HT department. | Daily | [same as above] | CEA India Grid EF [state year] |
| Bright Bar | Bright Bar Mfg. | Sub-meter reading — BBD department. | Daily | [same as above] | CEA India Grid EF [state year] |
| Wire Rod | Hot Rolling | Sub-meter reading — RMD department. | Daily | [same as above] | CEA India Grid EF [state year] |
| Wire Rod | Heat Treatment | Sub-meter reading — HT department. | Daily | [same as above] | CEA India Grid EF [state year] |
| Wire Rod | Pickling | Sub-meter reading — Pickling department. | Daily | [same as above] | CEA India Grid EF [state year] |
| Wire | Wire Manufacturing | Sub-meter reading — Wire department. | Daily | [same as above] | CEA India Grid EF [state year] |

In case of sub meters, describe reconciliation. The reconciliation Correction related data should also be collected and monitored. Also, In case of mix EF required, describe methodology/calculation here.

# Section 5: Precursors Used in Production

[Req. 14] Precursors used in each production process; supplier name and country of origin

A precursor is a CBAM good used as an INPUT material in another CBAM production process at the same or a different installation.

For each precursor, you must disclose: (a) which production process it feeds into, (b) whether it is produced internally (own plant) or externally (purchased), (c) if purchased externally: the supplier name and country of origin.

For internally transferred precursors (e.g. billets made in your SMS used in your rolling mill): write 'Internal transfer — [installation name]'.

This section feeds directly into the Specific Embedded Emissions (SEE) calculation for downstream goods.

## 5.1 Precursors Used in Production

| Sr. | Precursor Material | Used in Product | Production Process | How Quantity Measured | Supplier Name | Country of Origin |
| --- | --- | --- | --- | --- | --- | --- |
| HOW TO FILL: For internal transfers: Supplier = 'Internal — [installation name]'. For purchased: state actual supplier name. Attach full details as Annexure II. |  |  |  |  |  |  |
| 1 | Billet | Black Bars | Hot Rolling | [Describe measurement method] | Internal — [Installation Name] | [Country] |
| 2 | Billet | Wire Rod | Hot Rolling | [Describe measurement method] | Internal — [Installation Name] | [Country] |
| 3 | Black Bars | Bright Bars | Bright Bar Mfg. | [Describe measurement method] | Internal — [Installation Name] | [Country] |
| 4 | Wire Rod | Wire | Wire Manufacturing | [Describe measurement method] | Internal — [Installation Name] | [Country] |
| 5 | FeCr (Ferro Chrome) | Billet | SMS — EAF/AOD | Weighed at raw material area near AOD. | Refer Annexure II | Refer Annexure II |
| 6 | FeNi (Ferro Nickel) | Billet | SMS — EAF/AOD | Weighed at raw material area near AOD. | Refer Annexure II | Refer Annexure II |
| 7 | FeMn (Ferro Manganese) | Billet | SMS — EAF/AOD | Weighed at raw material area near AOD. | Refer Annexure II | Refer Annexure II |
| 8 | [Add precursor] |  |  |  |  |  |

## 5.2 —Precursor Supplier Details

This is mandatory for all externally purchased CBAM precursors. Internal transfers (precursors produced and used within the same installation) only need a one-line declaration — no supplier form required. For each external supplier: complete one row per precursor per supplier. If the same supplier provides multiple precursors, add a separate row for each. SEE Data: Ask your supplier whether they have their own CBAM monitoring plan and can provide a Specific Embedded Emissions (SEE) value for the precursor. If yes, record the value and its basis. If no, the CBAM default value from EU 2025/2620 will be used — state this explicitly. Do not leave this column blank. Best to ask data from supplier (agreement etc) before placing order toc check requirement, include CBAM requirements while placing the order to suppliers, ensure data flow from supplier quarterly basis as well as accredited verified data after one year.

| Sr. | Precursor Name & CN Code | Supplier Legal Name & Registration No. | Full Address & Country | Contact Person & Email | Approx. Annual Quantity (MT) | SEE Value Available? If yes: value (tCO2e/t) and basis. If no: state "CBAM default will be applied." |
| --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |

You need to maintain this template for each year for your CBAM verification and data collection. Template update frequency should be defined. (Suggested quarterly at least)

The internal transfer declaration (just a paragraph, no table needed):

The following precursors are produced and used within the same installation. No external supplier data applies. Embedded emissions for these precursors are calculated directly from the production process monitoring data documented in Section 3 of this Monitoring Plan.

List: [e.g. Billets transferred to Hot Rolling / Black Bars transferred to Bright Bar Manufacturing / Wire Rod transferred to Wire Manufacturing]

# Section 6: Mandatory Declarations

[Req. 12] Perfluorocarbons from primary aluminium production

[Req. 15] Zero-rated fuels

[Req. 16] Measurable heat import/export

[Req. 20] Waste gases

[Req. 21] CO2 capture, storage and/or use

Each declaration below is mandatory. Do not delete any row.

If a topic does not apply to your installation, explicitly state 'Not Applicable' and give a one-line reason.

Silence is NOT acceptable to a verifier. A blank section will trigger a non-conformance finding.

| Regulatory Topic | Declaration |
| --- | --- |
| Perfluorocarbons (PFC) from primary aluminium production (Req. 12) | NOT APPLICABLE. This installation produces iron and steel goods only. No primary aluminium production takes place at this installation. PFC emissions do not arise. |
| Zero-rated fuels (Req. 15) | No zero-rated fuels are used at this installation. All fuels (LSHS, LPG, LDO, PNG) are fossil-based and carry their full IPCC emission factors. [If you use biomass or other zero-rated fuels, state here: fuel name, quantity, and how zero-rating is demonstrated — e.g. sustainability certification] |
| Measurable heat import/export (Req. 16) | No measurable heat is imported from or exported to any other installation. All heat generated is consumed within this installation's production processes. |
| Waste gases produced/used/imported/exported (Req. 20) | No waste gases are produced, used, imported, or exported at this installation. [If applicable: identify the waste gas type, the other installation involved, and the monitoring method for attributing emissions to the waste gas flow] |
| CO2 capture, storage and/or use (Req. 21) | NOT APPLICABLE. No CO2 capture, storage, or utilisation takes place at this installation. No carbon is transferred to any receiving installation or transport infrastructure. |

This is by default declaration, in case of heat transfer or waste gas transfer this declaration is not required but details to be provided.

# Section 7: Product Quantity Measurement

For each CBAM good produced, document how the output quantity is measured.

This is the denominator in your Specific Embedded Emissions (SEE) calculation: SEE = Total Embedded Emissions / Quantity of Good Produced.

Accuracy of product measurement directly affects accuracy of your SEE — verifiers will cross-check this against production records.

Also note whether quality control / inspection is performed and how — this provides confidence in product identity.

| Sr. | Product | Production Process | How Output Quantity Measured | Measurement Freq. | Measurement Instrument & Location | QC Inspection? | QC Methodology |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Billet | SMS | Cast billets weighed on billet yard weighing scale. Recorded in register + SAP. | Heat-wise | Weighing scale — Billet Yard | Yes | Visual inspection of billet surface; Billet Macro Inspection for internal soundness. |
| 2 | Black Bars | Hot Rolling | Weighed during bundling at Finishing Yard. | Bundle-wise | Weighing scale — Finishing Yard | Yes | Visual, mix-up check, straightness, dimension, length. |
| 3 | Black Bars | Heat Treatment | Dispatchable bars weighed after inspection at Straightening Yard. | Bundle-wise | Weighing scale — Straightening Yard | Yes | Visual, straightness, dimension. |
| 4 | Bright Bars | Bright Bar Mfg. | Weighed during bundling. | Bundle-wise | Weighing scale — Bright Bar Dept. | Yes | Visual, straightness, dimension. |
| 5 | Wire Rod | Hot Rolling | Weighed on main weigh bridge for dispatch. Internal transfer: standard loss of 2.9% applied on billet weight. | Heat/wire/size-wise | Weigh Bridge — Near Admin | Yes | Visual, mix-up, dimension. |
| 6 | Wire Rod | Heat Treatment | Weighed on weigh bridge for dispatch. Standard 0.5% loss applied for internal transfer. | Heat/wire/size-wise | Weigh Bridge — Near Admin | Yes | Visual, mechanical properties. |
| 7 | Wire Rod | Pickling | All pickled coils weighed after pickling. | Per coil | Weighing scale — Pickling Dept. | Yes | Visual, dimension. |
| 8 | Wire | Wire Manufacturing | All wires weighed at Wire Dept. | Per coil | Weighing scale — Wire Dept. | Yes | Visual, dimension, mechanical properties. |

# Section 8: Measuring Equipment and Calibration Plan

[Reg. Req. Req. 22(a)] Quality assurance of measurement equipment — calibration, adjustment, and checking

List EVERY instrument used to measure any quantity relevant to emissions calculations and product quantity measurement (fuels, raw materials, products, electricity).

Calibration Status: State whether the instrument is currently within its calibration validity period. Yes / No / Overdue.

Calibration Frequency: How often is it calibrated? Annual, bi-annual, as per manufacturer, etc.

Accredited Calibration Organisation: The organisation performing calibration must be accredited (e.g. NABL-accredited lab in India).

Traceability: Calibration must be traceable to national/international measurement standards (e.g. NPL India).

If an instrument has NO calibration (e.g. purely theoretical calculation with no instrument), state 'No instrument — theoretical calculation' and justify why a meter cannot be installed.

IMPORTANT: 'No meter' for LPG in the SMS section is a weakness — verifiers may require you to install a meter or upgrade to a higher tier.

| Sr. | Instrument Name & Type | Source Stream Monitored | Production Process | Location | Calibration Status (Yes/No/Overdue) | Calibration Frequency | Calibrating Organisation (must be NABL/accredited) | Accreditation Standard |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HOW TO FILL: Fill every row completely. Empty calibration cells are a guaranteed non-conformance. If instrument has no calibration, state reason and plan to address. |  |  |  |  |  |  |  |  |
| 1 | Weighing Scale (Truck Weighbridge) | All incoming materials | Reception & Storage | Near Admin Block | [Yes/No] | [e.g. Annual] | [Enter org name] | [e.g. NABL accredited] |
| 2 | Scale-tube Level Meter | LSHS | SMS — Billet | On SMS sub-tank | [Yes/No] | [e.g. Annual] | [Enter org name] |  |
| 3 | Scale-tube Level Meter | LSHS | Hot Rolling | On main LSHS tank | [Yes/No] | [e.g. Annual] | [Enter org name] |  |
| 4 | Weighing Scale | LPG | Reception & Storage | Near Admin Building | [Yes/No] | [e.g. Annual] | [Enter org name] |  |
| 5 | No Meter — Theoretical Calc. | LPG | SMS | N/A | N/A | N/A — No instrument. Consumption derived by subtraction. Plan: [state if meter installation is planned] | N/A | N/A |
| 6 | Rotameter | LPG | Wire Manufacturing | Before Wire Mfg. section on pipeline | [Yes/No] | [e.g. Annual] | [Enter org name] |  |
| 7 | Optical Spectrometer | Scrap / Steel analysis | SMS — Lab | SMS Lab | [Yes/No] | [e.g. Annual] | [Enter org name] |  |
| 8 | LECO Combustion Analyser | Ferro-alloys / C content | SMS — Lab | SMS Lab | [Yes/No] | [e.g. Annual] | [Enter org name] |  |
| 9 | Electricity Sub-Meters (multiple) | Electricity consumption | All departments | SMS, RMD, HT, BBD, Wire, Pickling depts. | [Yes/No] | [e.g. Annual] | [Enter org name] |  |
| 10 | Billet Yard Weighing Scale | Billet product output | SMS | Billet Yard | [Yes/No] | [e.g. Annual] | [Enter org name] |  |
| [Add instrument] |  |  |  |  |  |  |  |  |

# Section 9: Data Quality Control System

[Reg. Req. Req. 22] Control system to ensure data quality — all sub-elements (a) through (h)

This is the most scrutinised section in a CBAM audit. Do NOT treat it as a formality.

The regulation requires you to demonstrate that data flowing from measurement through to the final reported emission figure is controlled at every step.

Complete all eight sub-sections (9.1 through 9.8). Each maps to one sub-element of Req. 22.

Be specific: name the person responsible, the system used (SAP, Excel, logbook), and the frequency of each control activity.

Generic statements like 'we have good internal controls' will be rejected.

## 9.1 Calibration Plan (Req. 22a)

Refer to Section 8 (Measuring Equipment and Calibration Plan) for the full calibration register. In addition, the following calibration controls are in place:

| Control Activity | Frequency | Responsible Person |
| --- | --- | --- |
| Review of calibration status of all instruments in the calibration register | Quarterly | [Enter name & designation] |
| Pre-use check of weighing instruments (zero/tare verification) | Daily before use | [Enter name & designation] |
| Calibration certificates filed and version-controlled | On receipt of certificate | [Enter name & designation] |
| Follow-up action if instrument found out of calibration | Immediately on identification | [Enter name & designation] |

Please describe in case of measurement failure or data gap, what methodology to be adopted?

## 9.2 Risk Assessment (Req. 22b)

Identify the key risks in your data flow where errors could occur between the primary measurement and the final reported emission figure.

For each risk: state the likelihood, potential impact on emission figures, and the control measure in place.

Typical risks for a steel plant: (a) meter failure or drift, (b) manual data entry errors from logbook to SAP, (c) incorrect carbon content values from lab, (d) missing data for external job-work, (e) allocation errors between co-produced goods.

| Sr. | Risk Description | Data Flow Step Affected | Likelihood (H/M/L) | Potential Impact on Emissions | Control Measure in Place |
| --- | --- | --- | --- | --- | --- |
| HOW TO FILL: Complete with at least 5 risks specific to your installation. Generic risks are insufficient. |  |  |  |  |  |
| 1 | Level meter drift leading to incorrect LSHS consumption reading | Activity data → SAP | Medium | Under/over-statement of combustion emissions | Annual calibration of level meters; daily cross-check of consumption against production volumes |
| 2 | Manual transcription error from logbook to SAP for LPG consumption | Primary data → ERP | Medium | Incorrect activity data for LPG combustion emissions | Supervisor review of SAP entries daily; monthly reconciliation vs. procurement |
| 3 | Incorrect or missing carbon content analysis for scrap batch | Calculation factor → emission calc | Low | Under/over-statement of mass balance emissions | Lab QC: duplicate analysis for each batch; Lab Incharge sign-off required before data entry |
| 4 | PNG consumption data not received from external job-work contractor | External data → monitoring plan | Medium | Missing emissions from outsourced heat treatment | Monthly consumption certificate mandatory from contractor; chased within 7 days of month-end |
| 5 | Incorrect electricity meter multiplier factor applied | Sub-meter reading → consumption calc | Low | Systematic error in indirect emission calculation | Multiplier factors verified annually against utility bill MF; discrepancies escalated to management |
| 6 | [Add risk specific to your installation] |  |  |  |  |

## 9.3 IT Systems Quality Assurance (Req. 22c)

Describe how your IT systems (ERP, Excel, databases) that handle emissions data are controlled.

At minimum address: access controls (who can enter/edit data), version control for calculation spreadsheets, backup procedures, and validation checks built into the system.

If SAP is your ERP: describe which SAP module stores the data, who has edit rights, and how errors are corrected.

| IT System | Control Measures | Responsible Person |
| --- | --- | --- |
| SAP ERP — Production & inventory data | Data entered by designated department operators only. Edit rights restricted by SAP user profile. Changes require supervisor approval. Monthly data lock after reconciliation. | [Enter SAP Administrator name] |
| Logbooks (physical) | Pre-printed formats with date/signature columns. Stored for minimum 5 years. Scanned copies retained digitally. | [Enter name] |
| CBAM Calculation Spreadsheet | Version-controlled (filename includes version number and date). Only CBAM Coordinator has edit rights. Read-only copies shared with management. Formula cells locked. | [Enter name] |
| [Add other system if applicable] |  |  |

## 9.4 Segregation of Duties (Req. 22d)

The person who collects primary data should NOT be the same person who enters it into the final emission calculation.

Describe who is responsible at each stage of the data flow: (1) Primary measurement, (2) Data recording, (3) Data entry to ERP, (4) Emission calculation, (5) Internal review, (6) Final sign-off.

If your organisation is small and some roles must overlap, acknowledge this and state the compensating control.

| Data Flow Stage | Role / Designation | Compensating Control if Dual Role |
| --- | --- | --- |
| Primary data measurement (instrument readings, logbooks) | [e.g. Shift Operator / Fitter] | N/A |
| Data recording in logbook / register | [e.g. Shift Supervisor] | N/A |
| Data entry into SAP | [e.g. Department In-charge] | N/A |
| CBAM emission calculation preparation | [e.g. CBAM Coordinator / Environment Manager] | N/A |
| Internal review and validation | [e.g. Finance / Management Representative] | N/A |
| Final approval and sign-off of monitoring plan | [e.g. Plant Manager / Director] | N/A |

This can be kept as annexure and up to date for each and every activity.

## 9.5 Internal Reviews and Data Validation (Req. 22e)

| Review / Validation Activity | Frequency | Responsible Person |
| --- | --- | --- |
| Monthly reconciliation of fuel consumption (SAP vs. procurement invoices) | Monthly | [Enter name] |
| Monthly reconciliation of production quantities (SAP vs. dispatch records) | Monthly | [Enter name] |
| Quarterly review of CBAM emission calculations for completeness and accuracy | Quarterly | [Enter name] |
| Annual internal audit of CBAM monitoring plan vs. actual practice | Annual | [Enter name] |
| Cross-check of lab analysis results against reference standards | Per batch analysis | [Enter name — Lab Incharge] |

## 9.6 Corrections and Corrective Actions (Req. 22f)

Describe the procedure for identifying, documenting, and correcting errors in emission data.

A corrective action procedure must exist — not just 'we fix errors when we find them'.

Include: how errors are reported, who authorises a correction, how corrections are documented in SAP/logbooks, and whether historical data is restated.

The following corrective action procedure applies to all CBAM monitoring data:

| Step | Action | Responsible Person | Documentation |
| --- | --- | --- | --- |
| 1 | Error identified (by internal review, audit, or verifier) | [Enter name] | Error log entry in CBAM register |
| 2 | Root cause analysis — determine whether error affects calculated emissions materially | [Enter name] | Written root cause note |
| 3 | Correction applied in SAP / calculation sheet with clear audit trail | [Enter name] | SAP correction document; updated calculation version |
| 4 | Corrective action to prevent recurrence — e.g. additional training, meter upgrade, process change | [Enter name] | Corrective action record with target completion date |
| 5 | Verification that corrective action was implemented | [Enter name] | Sign-off in CBAM register |

## 9.7 Control of Outsourced Processes (Req. 22g)

If any step in your production process is outsourced to an external installation, you remain responsible for the emissions from that step.

For each outsourced process: describe how you collect and verify the emissions data from the contractor.

The data collection method must be equivalent in quality to what you would do in-house.

Common example for steel plants: heat treatment at job-work facilities using PNG.

| Outsourced Process | External Installation Name & Location | Fuel / Emission Source | How Data Collected from Contractor | Verification Method |
| --- | --- | --- | --- | --- |
| HOW TO FILL: If no outsourced processes: write 'No production processes are outsourced to external installations.' If applicable, fill all columns. |  |  |  |  |
| Heat Treatment of Black Bars | [Enter contractor name, city, state] | PNG (Piped Natural Gas) | Monthly consumption certificate provided by contractor within 7 days of month-end. Format: quantity (MT or Nm3), period, signed by plant manager. | Cross-check against production quantities sent for job-work. Flag if consumption/tonne deviates >15% from historical average. |
| [Add if applicable] |  |  |  |  |

Can keep a provision of 3/ 6 monthly internal audit of each of the outsourced partner

## 9.8 Record-Keeping and Document Version Management (Req. 22h)

The regulation requires you to retain all monitoring records and supporting documentation.

Minimum retention period: consistent with the CBAM declaration period + 4 years (follow your legal counsel's advice for the specific retention period applicable).

Document version management: every update to the monitoring plan must be captured with a version number, date, and brief description of what changed.

| Document / Record Type | Storage Location | Retention Period | Responsible Person |
| --- | --- | --- | --- |
| Monitoring Plan (all versions) | CBAM document folder (digital) + physical binder | Minimum 5 years | [Enter name] |
| Logbooks — fuel consumption, production | Department logbooks (physical) + scanned copy on server | Minimum 5 years | [Enter name] |
| SAP production and inventory reports | SAP system + monthly backup to server | Minimum 5 years | [Enter IT / ERP admin name] |
| Calibration certificates | Calibration register binder + digital folder | Life of instrument + 2 years | [Enter name] |
| Lab analysis certificates (carbon content) | Lab register + scanned copy on server | Minimum 5 years | [Enter Lab Incharge name] |
| Contractor consumption certificates (PNG job-work) | CBAM document folder | Minimum 5 years | [Enter name] |
| CBAM emission calculation workbooks | Version-controlled folder on server (read-only after finalisation) | Minimum 5 years | [Enter CBAM Coordinator name] |

## 9.9 Document Version History

| Version | Date | Changes Made | Approved By |
| --- | --- | --- | --- |
| 01 | 01/01/2026 | Initial creation of Monitoring Plan | [Enter name] |
| [02] | [DD/MM/YYYY] | [Describe changes] | [Enter name] |

# Annexure Reference

The following documents must be attached to and submitted with this Monitoring Plan:

| Annexure | Title | Description / What to Include |
| --- | --- | --- |
| I | System Boundary Diagram | Diagram showing: (a) physical installation boundary, (b) individual production process boundaries, (c) all material/energy inputs and outputs with labels, (d) annotation confirming no double-counting and no data gaps. Must be drawn by a qualified engineer and signed off by plant management. |
| II | Precursor Supplier Data | For all externally purchased CBAM precursors: company name, registration number, full address, country, contact person, product supplied, and approximate annual quantity. For ferro-alloy suppliers: attach mill certificates showing carbon content. |
| III (if applicable) | Calibration Certificates | Copies of current calibration certificates for all instruments listed in Section 8. Particularly: weighbridges, level meters, rotameter, spectrometer, LECO analyser. |
| IV (if applicable) | Electricity Emission Factor Documentation | Copy of CEA grid emission factor publication used, with the specific value and reporting year highlighted. |

DECLARATION: The undersigned confirms that the information contained in this Monitoring Plan is complete, accurate, and compliant with the requirements of Commission Implementing Regulation (EU) 2025/2547. This plan will be updated promptly if any material changes occur at the installation.

| Name: ___________________________________ Designation: ____________________________ Date: ___________________________________ | Signature: ______________________________ Company Seal: |
| --- | --- |

 Yellow boxes = instructions, delete before submitting to verifier

 Green boxes = examples, replace with your own data

 Annexure I (system boundary diagram) is the one thing, but a process flow won't substitute